# Sensorius — Mathematical Reference

Units, algorithms, and output definitions for Sensorius. Each analysis mode includes a summary, step-by-step procedure, and formulas as implemented in sensor_standalone. Persistence describes the portable SQLite project model.

# 1. Units and conventions

| Quantity | Storage / UI | In calculations |
| --- | --- | --- |
| Depths | metres (m) | metres |
| Scope, core length, trigger core length | feet (ft) | Lₘ = Lft ÷ 3.28 |
| Trigger line length (Alter) | feet (ft) | Lₘ = Lft ÷ 3.28 |
| Trigger penetration | metres (m) | metres |
| Time offsets | seconds (s) | seconds |
| Sampling rate | hertz (Hz) | fₛ = 1 / median(Δt) |
| Weight-stand length | — | 1.5 m (fixed) |
| Piston offset constant | metres (m) | default 1.25 m |

Depth increases downward (oceanographic convention).

FT_TO_M = 1/3.28 throughout (not 0.3048).

Indices refer to merged sensor dataframe rows.

NaNs are interpolated before filtering or geometry.

# 2. Preprocessing (Data Manager)

Summary

Parse RBR/Star-Oddi files, keep channels matching depth/pressure/acceleration, prefix columns by sensor location and filename, outer-merge on datetime.

Algorithm

Parse each file → datetime + numeric columns.

Rename: {location}_{filename}_{column}.

Outer merge on datetime; sort ascending.

Depth columns contain 'Depth' in the name.

Winch auto-load

Select winch files overlapping core deployment window.

Concatenate, sort by datetime, trim to deployment window.

Manual winch mode loads full files without trimming.

# 3. Analysis modes

## 3.1 View Data

Summary: Display merged depth time series.

Outputs: Visual only.

## 3.2 Depth Offset

Summary: Depth-dependent calibration regression and/or manual offsets.

Regression

Δj,i(d) = slope · d + intercept

Δj,i = predicted (sensorⱼ − sensorᵢ) at depth d.

dtarget,corr = dtarget + sign · Δj,i(dref)

| Reference | Sign | Action |
| --- | --- | --- |
| sensorᵢ in pair (j−i) | +1 | Add offset to target i |
| sensorⱼ in pair (j−i) | −1 | Subtract offset from target j |

Manual offset

dcol,corr = dcol + δm

δₘ in metres.

## 3.3 Time Offset

Summary: Heave cross-correlation for clock alignment.

Heave: 2nd-order detrend, then 4th-order Butterworth bandpass 0.05–0.5 Hz.

c[k] = Σnhref[n] · hmov[n−k]

k* = argmax |c[k]|

δ = ½ · c[k*−1] − c[k*+1]c[k*−1] − 2c[k*] + c[k*+1]

Δt = (k* + δ) / fs

Apply via linear interpolation: z_corrected(t) = interp(t, t+Δt, z).

## 3.4 Create Calibration

Summary: ≥2 casts → linear regression of pairwise differences vs mean depth.

d̄all = meanₙ(meanᵢ dᵢ[n])

Δ̄j,i = meanₙ(dⱼ[n] − dᵢ[n])

Δ̄j,i = slope · d̄all + intercept

## 3.5 Trip Detector

Summary: Trip when cross-sensor derivative spread exceeds threshold.

d̂i(k)[n] = Savgol(dᵢ, W, P, deriv=k, Δt)

Defaults: W=51, P=3, k=1 (velocity), fₛ=32 Hz, τ=0.5, edge buffer=500.

σ[n] = stdᵢ( 
                d̂i(k)[n] )

Mask first/last 500 samples.

If σ[n] > τ: trip_idx = first n (confidence 0.8).

Else: trip_idx = argmax σ (confidence 0.5).

## 3.6 Piston Position

Start-core (Sₘ = scopeft / 3.28):

start_core = min{n ≥ trip : |drelease[n] − dWS[n]| > Sm}

Piston trace (Bₘ = core_lengthft / 3.28, O = 1.25 m default):

dpiston[n] = dWS[n] + O + Bm

dpiston[n] = drelease[n] + Sm + Bm + O   (from start_core onward)

## 3.7 Winch

Trip: first w[n−1] − w[n] ≥ 1000 in search window (edge buffer 500).

Δtwinch = tsensor − twinch,orig

Twinch,aligned[n] = Twinch,orig[n] + Δtwinch

## 3.8 Calculate

Seafloor:

dseafloor = dtrigger[trip] + LTC,m − ptrigger

Core tip & start_pen:

dtip[n] = dWS[n] + 1.5 + Lcore,m

start_pen = min{n ≥ trip : dtip[n] ≥ dseafloor}

| Output | Formula | Requires |
| --- | --- | --- |
| recoil_max | ¦d_rel[trip] − min d_rel[trip…end_pen]¦ | trip |
| recoil_time_s | (n_peak − trip) × median(Δt) | trip |
| fall_dist | ¦d_WS[trip] − d_WS[end_pen]¦ | trip |
| suck_in | ¦d_WS[end_pen] − max d_WS¦ | trip |
| recoil_start | ¦d_rel[trip] − d_rel[start_core]¦ | trip + start_core |
| freefall_start | ¦d_WS[trip] − d_WS[start_core]¦ | trip + start_core |
| piston_suck | d_piston[pullout] − d_piston[cross] | piston |
| pen_deficit | d_seafloor − max d_WS | seafloor |
| piston_alt | d_seafloor − d_piston[start_core] | seafloor |
| freefall_est | ¦d_WS[trip]+1.5+L_core,m−d_seafloor¦ | seafloor |

## 3.9 Alter

Summary: Re-run piston + Calculate with edited rig parameters; trip/end_pen/pullout fixed.

Δtrig,m = (trigger_line_ft_new − trigger_line_ft_db) / 3.28

ΔTC,m = (trigger_core_length_ft_new − trigger_core_length_ft_db) / 3.28

dWS,mod = dWS − Δtrig,m − ΔTC,m

Similarly: d_release,mod and d_trigger,mod with the same shifts.

Pig-weight changes: not implemented.

## 3.10 Piston Analysis

xcore[n] = dWS[n] − dWS[start_pen]

v[n] = Savgol(d, W, P, deriv=1, Δt)   [m/s],  W=51, P=3

# 4. Save, restore, and persistence

Project folder: sensorius_data.sqlite + sensor_data/<cruise>/ + winch_data/<cruise>/. Multiple analysis_results rows per core.

analysis_results flat columns

| Column | Meaning |
| --- | --- |
| trip_index, trip_datetime | Sensor trip event |
| start_core_index | Start of coring |
| result_* | Calculate outputs (m; recoil_time_s in s) |
| trigger_pen_m | Trigger penetration input (m) |
| winch_trip_index, winch_time_offset_sec | Winch alignment |
| params | JSON: depth_offset, time_offset, trip, piston, calc_inputs, winch, … |

Restore order

Depth corrections → Time corrections → Trip → Winch → Piston → Calculate

Skipped sections reported in status bar.

trigger_core_penetration written back to core row on save.

# 5. Savitzky–Golay filter

x̂[n] = Σ cj x[n+j]

Local polynomial least-squares (SciPy savgol_filter).